# NexaTag AI+ — ESP32 RFID Access Control System

NexaTag AI+ is a smart RFID-based access control system built using the ESP32 microcontroller and RC522 RFID module. The system authenticates RFID cards and provides visual and audio feedback through an RGB LED and buzzer.

## Project Features

RFID card authentication using RC522

ESP32-based embedded control system

RGB LED status indication

Audio alerts using buzzer

PCB-ready hardware design

Fusion 360 / KiCad PCB workflow

Expandable IoT architecture

## Components Used

| Component | Purpose |
| --- | --- |
| ESP32 DevKit | Main microcontroller |
| RC522 RFID Module | RFID card reader |
| RGB LED | Access status indication |
| Buzzer | Audio alert system |
| 220Ω & 330Ω Resistors | Current limiting |
| USB Power | System power supply |

## Circuit Connections

### RC522 RFID Module → ESP32

| RC522 Pin | ESP32 GPIO |
| --- | --- |
| SDA | GPIO5 |
| SCK | GPIO18 |
| MOSI | GPIO23 |
| MISO | GPIO19 |
| RST | GPIO27 |
| 3.3V | 3.3V |
| GND | GND |

### RGB LED → ESP32

| LED Pin | ESP32 GPIO |
| --- | --- |
| Red | GPIO25 via 220Ω |
| Green | GPIO26 via 220Ω |
| Blue | GPIO14 via 330Ω |
| Common Cathode | GND |

### Buzzer → ESP32

| Buzzer Pin | ESP32 GPIO |
| --- | --- |
| Positive (+) | GPIO13 |
| Negative (-) | GND |

## Working Principle

ESP32 continuously monitors the RC522 RFID module.

RFID card UID is detected when the card approaches the reader.

ESP32 authenticates the UID.

Green LED indicates authorized access.

Red LED indicates unauthorized access.

Buzzer provides audio feedback.

## PCB Design Workflow

Schematic creation

Component placement

PCB routing

Ground plane generation

Design Rule Check (DRC)

Gerber file export

## Applications

Smart access control systems

Attendance monitoring systems

Office security

Smart home authentication

IoT-enabled identification systems

## Future Improvements

Wi-Fi logging system

Mobile application integration

Cloud database connectivity

Solenoid lock integration

OLED display support

Fingerprint authentication

Battery backup support

## Conclusion

NexaTag AI+ combines RFID technology, embedded systems, and PCB design into a compact smart authentication platform suitable for competitions and IoT applications.

## Developed By

Team VOLTIX